Fire Safety Log Book

Use this log book to record fire safety checks, tests, servicing, drills and training at your premises. Keeping records shows you're maintaining your fire precautions and helps you spot problems early.

Under the Regulatory Reform (Fire Safety) Order 2005, the responsible person must make sure fire safety equipment and facilities are maintained in efficient working order and good repair, and that staff are given suitable training. Keep this log book somewhere safe and easy to find, and show it to your fire risk assessor, insurer or fire and rescue service when asked.

Premises and contacts

| Premises |  | Log book started |  |
| --- | --- | --- | --- |
| Responsible person |  | Phone |  |
| Fire alarm company |  | Phone |  |
| Extinguisher company |  | Phone |  |
| Emergency lighting company |  | Phone |  |

Recommended checks

Typical frequencies based on British Standards (BS 5839-1, BS 5266-1, BS 5306-3) and government guidance. Follow your fire risk assessment and your equipment suppliers' advice where they differ.

| Check | How often | Who |
| --- | --- | --- |
| Fire alarm test | Weekly, using a different call point each time in rotation | Trained staff member |
| Fire alarm service | At least every 6 months | Competent fire alarm engineer |
| Emergency lighting test | Monthly short functional test | Trained staff member |
| Emergency lighting full test | Annually, for the full rated duration (usually 3 hours) | Competent person |
| Fire extinguisher check | Monthly visual check | Trained staff member |
| Fire extinguisher service | Annually | Competent service engineer |
| Fire doors and escape routes | Weekly or monthly, depending on use | Trained staff member |
| Fire drill | At least once a year, more often for high staff turnover or shifts | Responsible person or fire marshal |
| Fire risk assessment review | Regularly, at least annually and after any significant change | Responsible person or competent assessor |

Fire marshals and wardens

| Fire marshal / warden | Area covered | Date trained | Refresher due |
| --- | --- | --- | --- |

1  Weekly fire alarm tests

Activate a different call point each week and check the alarm sounds everywhere. Warn people before you test.

| Date | Call point / zone tested | Result (OK / fault) | Action taken | Tested by |
| --- | --- | --- | --- | --- |

2  Fire alarm servicing

| Date | Engineer / company | Faults found and action taken | Next service due |
| --- | --- | --- | --- |

3  Emergency lighting

Monthly: switch to battery for a short time and check every light works. Annually: full-duration test by a competent person.

| Date | Type of test | Result | Faults and action taken | Tested by |
| --- | --- | --- | --- | --- |

4  Fire extinguishers

Monthly: check each extinguisher is in place, visible, undamaged, with the pin and seal intact and the gauge (if fitted) in the green. Annually: service by a competent engineer.

| Date | Check or service | All OK? | Faults and action taken | Checked by |
| --- | --- | --- | --- | --- |

5  Fire doors and escape routes

Check fire doors close fully and aren't wedged open, seals and signs are intact, and escape routes and final exits are clear and open easily.

| Date | Doors and routes checked | All OK? | Faults and action taken | Checked by |
| --- | --- | --- | --- | --- |

6  Fire drills

| Date | Time | Time to clear building | Number evacuated | Problems found and action taken | Led by |
| --- | --- | --- | --- | --- | --- |

7  Fire safety training

| Name | Training (e.g. induction, fire marshal, extinguishers) | Date | Trainer |
| --- | --- | --- | --- |

8  Defects and actions

Record any fault or fire safety issue found at any time, and the date it was fixed.

| Date found | Defect or fire safety issue | Action by | Date fixed | Signed |
| --- | --- | --- | --- | --- |

Need a fire risk assessment or fire marshal training? 3Sixty's chartered consultants can help. Call 01869 222 144 or visit 3sixtyhealthandsafety.com.

This log book is a guide. Follow your fire risk assessment and the maintenance advice for your own equipment. Print extra pages as needed.